78 GIRI

INTRODUCTION

It is now forty years since 78 rpm records were discontinued and it is becoming increasingly difficult to find equipment to play them. Most hi-fi turntables don't even run at 78rpm, and few cartridges are obtainable with suitable styli. Even if you can get hold of a player, none of the readily available equipment addresses the four major difficulties: stylus size and shape, speed,equalization, and surface noise. This article gives brief notes on each aspect. I have provided some demonstration transfers in mp3.

BASIC TURNTABLE

It is now quite difficult to find turntables capable of playing 78s at all, let alone the more specialized ones needed to do the job properly. If you just want one which will run at 78rpm without the speed variations a good bet would be the Pro-ject Debut, available from Henley Designs Ltd (on their home page, click on the link for 'Pro-ject'). The basic 33/45 turntable with arm and cartridge costs £110, and there is an optional additional kit for playing 78s - around £30 for the stylus and around £20 for the additional speed (the turntable is belt driven and the kit gives you an extra pulley). The stylus is likely to be fairly basic and you might want to consider upgrading it as described in the next section.

STYLI

At the time the format disappeared the recommended stylus size was 2.5 thousandths of an inch conical tip. This would be satisfactory for records made in the final few years, but for the great majority of the time since the beginning of 78s in 1896 the grooves were wider than the 'standard', with the result that a 2.5 thou stylus skates about in the bottom of the groove, seriously increasing the surface noise.

(I have written a separate note on stylus tracking with illustrations.)

The simplest solution is a 3 thou. conical stylus. This will play older records quite well, although end-of-side distortion is quite apparent; but it doesn't cope well with latter-day pop records, particularly since these are usually recorded at a very high level.

I have obtained the best results with a truncated elliptical tip having a major radius of 2.8 thou. The narrower minor radius enables the stylus to track the grooves with much less distortion, while the major radius allows the contact points to be well up the groove walls away from the muck in the bottom.

In order to obtain one you will have to find a firm who can cement a specially made tip onto an existing cartridge. This can come a bit expensive, but since a modern cartridge tracks the records at an extremely light weight, the wear is almost non-existent and the stylus should last for a very long time.

For readers in the U.K.,I can recommend a firm called 'The Cartridge Man', who have re-tipped stereo cartridges for me and can also do 78 styli, including advising on the best stylus for your needs. They now have a web site, and their address is:

SPEEDS

By no means all 78s were actually recorded at 78 rpm. Even in the late 1920s English Columbia was still using 80 rpm, and prior to about 1921 speeds were widely variable. Some of the audio tracks included in the Music hall section of this site were transferred at speeds as low as 74 rpm, and I have come across records where the speed was as low as 68 or as high as 84 rpm.

To make matters worse, relatively few records state the speed (and when they do it's not always accurate). If the work is a classical piece such as an opera aria, it is possible to check the correct pitch against a score or a modern recording: but as occasionally singers would transpose, this isn't completely reliable. My best recommendation is to gradually reduce the speed of a record until it starts to sound sluggish, and then increase it slightly (in my experience the ear is much more sensitive to low speeds than high speeds). I'm afraid it's all rather rule of thumb: and the narrow bandwidth of old records doesn't make it any easier.

As to a turntable capable of coping with these speeds, that is yet another problem. Few turntables have more than a tiny variation (usually 2 or 3 per cent, which is nowhere near enough); but electronically controlled turntables may be modifiable. You need a speed range of 72 to 82 to cover most records. I'm using a Goldring-Lenco turntable which has a mechanical system for continuously varying the speed from about 32 to about 84 rpm, but it's not available any more - indeed I have had mine since 1963!. (For anyone who needs Goldring -Lenco spares I have given an address on another page). Vestax, who specialize in disco equipment, make the BDT 2000 turntable which has speeds from 16 to 98 rpm (though their UK web site doesn't mention it): the cost is around £190/$450. Vestax's UK address is 18 St. Christopher's Road, Haslemere, Surrey GU27 1DQ, telephone +44 (0)1428 653117.

EQUALIZATION

(I have written a separate technical explanation of the reason for equalization)

As with the styli, the 'standard' reproduction curve was set in the mid-1950s, and is wrong for most of the history of 78s! Using tone controls doesn't really get it right because they don't reach the middle of the frequency range. I list here the principal curves for 78s, and, since I have them available, those for early LPs before RIAA(CCIR) became the standard.

The curves follow much the same pattern: a treble cut above a specified frequency; a bass boost below another frequency, and sometimes a flattening of the extreme bass. Early electrical 78s have a flat upper frequency response. Note that these are the reproduction curves: the recording curves would of course have been the inverse (allowing for losses in the head mechanism, etc.).

I have provided a preamplifier circuit with the following curves:

| COARSE GROOVE ('78 rpm') |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SYSTEM | Treble turnover | Bass turnover | Lower bass t/o | Cut at 10 kHz | Boost at 50 hz |
| DECCA 78 | 3.4 kHz | 150 Hz | - | 9 dB | 11 dB |
| ffrr 78 | 6.36 kHz | 250 Hz | 40 Hz | 5 dB | 12 dB |
| WESTREX | flat | 200 Hz | - | - | 15 db |
| BLUMLEIN | flat | 250 Hz | 50 Hz | - | 12 dB |
| BSI 78 | 3.18 kHz | 353 Hz | 50 hz | 10.5 dB | 14 dB |
| MICROGROOVE (lps and 45 rpm) |  |  |  |  |  |  |  |  |  |  |
| SYSTEM | Treble turnover | Bass Turnover | Lower bass t/o | Cut at 10 kHz | Boost at 50 hz |
| RIAA (CCIR) | 2.1215 kHz | 500.5 Hz | 50.5 Hz | 13.6 dB | 17 dB |
| ffrr LP | 3 kHz | 500 Hz | 100 hz | 10.5 dB | 12.5 dB |
| EMI LP | 2.5 kHz | 500 Hz | 70 Hz | 12 dB | 14.5 dB |
| NARTB | 1.6 kHz | 500 Hz` | - | 16 dB | 16 dB |
| COLUMBIA | 1590 Hz | 500 Hz | 100 Hz | 16 dB | 12.5 dB |

Jørgen Vad manufactures a preamplifier which provides equalization curves: details from his web site.

RIAA (CCIR is identical) is the current standard for all microgroove records,ffrr LP for pre-1955 English Decca LPs, EMI LP for pre-1955 HMV and English Columbia LPs, COLUMBIA for older American Columbia LPs, NARTB for some early American labels.

78s: WESTREX (English Western Electric) for HMV 78s with triangle matrix code and English Columbias with a matrix code. BLUMLEIN for HMVs with a square by the matrix number, and English Columbias with a , or in both cases with no code (post 1945) up to about 1953. BSI 78 for all post 1953 78s (in theory). HMVs withare American Victor recordings: use WESTREX.

SURFACE NOISE

So, you've got the right stylus, the right speed, and the right equalization, and you play the record. It should sound fine, but there is still one problem - surface noise, always the great bugbear of 78s. There are three principal causes:

Earlier records had an abrasive filler added to the shellac mix from which they were made, with the idea of grinding down the steel or fibre (cactus thorn) needles to fit the groove shape

Wear and damage - steel needles were very hard on the groove and a badly worn record could look quite grey. Add scratches or even splits to that and you can have a major problem.

Granulation: some makes of records - inter-war HMVs are particularly bad - are prone to bacterial attack from bacteria living in the cardboard sleeves (particularly if they get damp) which causes the shellac to harden in little granules and increase the noise, even on a mint record.

What can you do about it? Firstly, try to get a good copy! - easier said than done. In the case of HMVs, you will get better results from Indian or Australian pressings than British ones. Some makes are markedly better - British Columbias are usually very good, using a finer filler to reduce the noise (or at least move it up in frequency where it is less of a problem). If you find an inter-war HMV which hasn't granulated, keep it dry, preferably in a polythene sleeve.

After that, you have only three choices:

Ignore the surface noise. That is what I do - you get used to it if it isn't too horrendous, but I am aware that for many people this is difficult. Incidentally if you are dubbing to cassette, you have to reduce the surface noise, because the tape simply won't cope with the very high levels at high frequencies (another equalization problem).

Filter it - reduce the top, preferably with a fairly steep cut filter rather than an ordinary tone control. I use a filter circuit reverse-engineered from a Quad 33 preamplifier, which provides separate adjustment of turnover frequency and slope, and is very flexible. Something along these lines is very useful if you can find it. Most LP or CD issues of 78s, at least until recently, filter records heavily, thus removing a good deal of the life from the recording together with the surface noise; what is needed is a compromise. Ideally use as little filtering as you can stand.

Noise reduction: in the last few years increasingly sophisticated equipment has become available for reducing crackle and clicks without affecting the actual signal. Unfortunately they are all extremely expensive, sufficiently so to be the province of the professional studio rather than even the well-off amateur. The Packburn is an analogue device which in effect compares the two groove walls moment by moment and chooses the quieter. The more recent Cedar system applies digital processing to the signal, analyzing it to decide which is signal and which is noise. Carefully set up it can produce quite stunning results - the noise almost disappears, even on alarmingly noisy originals. I wish I could afford one -but they cost quite some thousands of pounds (I don't know how much - it's rather a case of if you have to ask you can't afford it).

In the unlikely event that I have any rich readers, here are the contact addresses for Packburn and Cedar.

Packburn Audio Noise Suppressor (Model 323A) US $2650: A modified PSI preamplifier containing three processors for reduction (dynamic filtering) of steady-state noise and "de-clicking" of transient noises, plus special features for optimum reproduction of old records, lateral and vertical. Widely used in the record industry.

STYLUS TRACKING

The basic stylus is conical, with a spherical tip; the included angle of the cone is 60 degrees. Most modern styli are elliptical: i.e. they look the same from the front, but from the side they have a much narrower angle. This enables them to track movements in the groove more easily (as the groove moves from side to side it effectively narrows - not across its direct width but across the diameter of the conical tip, at an angle to the direction of travel: consequently a spherical tip will ride up in the groove).

The illustrations below could apply equally to a spherical tip and an elliptical tip seen from the front: in practice I would always recommend elliptical.

A modern record groove should be a neat 90 degrees, with the stylus making contact at only two points part-way up the groove wall, thus avoiding any debris in the bottom of the groove. (Some modern microgroove styli have a flat section at the contact point, giving a wider area of contact which reduces wear: however it can also increase noise from worn or damaged groove walls and I would be dubious about its use for 78s).

In the case of microgroove records the specification for the stylus tip is 1 thousandth of an inch for mono, or 0.7 thou. for stereo (with variations for ellipticals). For 78s the mid-1950s specification is 2.5 thou, and this works well enough in a properly cut groove. However older 78s were cut to be used with steel needles, on the assumption that the needle would wear down to fit the groove in the first few seconds: when this wider groove is tracked with a 2.5 thou stylus the result is 'bottoming' as can be seen here.

Here the combination of a larger basic size - 2.8 thou. works on the majority of records, although larger sizes may be helpful with very old records - and the truncated tip (which is spherical down to the contact points but truncated below) result in vastly better tracking. If the stylus is elliptical this also brings a considerable improvement in distortion.

PREAMPLIFIER CIRCUIT FOR EQUALIZATION OF 78rpm RECORDS (and older microgrooves)

I enclose two gifs of the main circuit, and the feedback loop. I'm afraid the image quality is less than perfect: they were taken from a photocopy of a hand-drawn circuit, which I scanned (using my fax machine!) and converted to a gif. However I think you will be able to read all the component values, which is what matters. As far as I know all the components are still readily available, except that one or two elecrolytic capacitor values are no longer made - just use the next highest.

No warranty is expressed or implied as to the suitability of this circuit for this or any other purpose. I regret I can't offer technical support or answer technical questions about this circuit.

EQUALIZATION OF GRAMOPHONE RECORDS

The simplest form of disk-cutter consists of a amplifier, similar to that used to drive a loudspeaker, connected to a cutting-head having a stylus connected to a coil, which is placed in the field from a strong magnet (or, more usually in later designs, a magnet within a coil). When the signal is applied to the coil, the stylus moves and engraves a groove in the blank disk. (There is of course a lot more to it than that, but we are considering only the basics here.)

However, because the cutter head's movements translate the amplitude swings of the original signal into velocity - the rate at which the stylus moves during its swings - low-frequency signals would be recorded with a much larger swing than high-frequency signals of the same original amplitude. In order to keep the movements of the groove much the same at all frequencies (given equal level signals) it is necessary to use a circuit to introduce - in the theoretical situation - a 6dB/octave cut as the frequency decreases - i.e., halve the frequency and you halve the voltage.

In the reverse situation, that of a reproduction head, the principal is that of a wire moved in a magnetic field - it is the rate of cutting 'lines of force' that matters. The cutter head works exactly in reverse, like a simple motor, where increased voltage means increased speed. Therefore the constant amplitude groove theoretically achieved produces a signal where the bass is low and the treble high: so a 6db/octave cut with increasing frequency would be called for.

In the real world, losses in the head with increased frequency complicate the issue. Early cutter heads were highly inefficient, and so, while the bass cut described above was used, the treble tailed away, resulting in equal groove modulations (movements) up to mid frequencies, but decreasing above that.

To compensate for this, the playback characteristic boosted the bass below 200 Hz but left it flat above that - effectively providing a 6dB/octave boost to the higher frequencies (and the surface noise). With the later improvements in cutters, it was possible to pack more treble onto the records, and so new equalizations provided for a 6dB/octave cut above a turnover frequency which varied between 3.4 and about 6kHz depending on the system. This meant that the surface noise became less obtrusive. It was also common to flatten out the bass at the very lowest frequencies to reduce the boost of rumble from the turntable.

Similar techniques were applied to microgroove records, and the final standard, RIAA (CCIR in Europe is the same) provides for a bass boost below 500.5 Hz and a treble cut above the lower frequency of 2.2125kHz - the latter reflecting the considerably increased amount of treble which can be cut onto an LP.

It can therefore be seen that playing a 78 with RIAA equalization - all that is available to many people - produces far less top than is correct -particularly for the earliest electrical, where the result is akin to turning the treble control right down. (Turning it right up gives an improvement, but doesn't touch the important mid-range.)

Use of the correct curves when reproducing 78s produces a startling improvement in the quality (although admittedly the surface noise can become a problem): many of these recordings are much higher quality than you might suppose.